# Blame Assignment for Dynamic IR-Drop in LTI RC Power Distribution Networks

PDN Analysis Team

March 2026

Abstract

This report derives the method for decomposing dynamic IR-drop at a victim node into exact per-instance aggressor contributions under a linear time-invariant RC model. The primary derivation uses the impulse response (Green’s function) of the RC network: by superposition, the victim’s droop is a convolution of each source’s current waveform with the network’s sensitivity kernel. A single backward-in-time evaluation of this kernel yields all aggressor contributions simultaneously. The Lagrangian adjoint formulation, which generalizes to nonlinear and time-varying systems, is presented in an appendix along with the spectral interpretation.

# 1. Introduction and Problem Statement

## 1.1 The Blame Question

Consider a power distribution network (PDN) modeled as an RC mesh with N nodes, P voltage-source pads held at Vdd, and M time-varying current sources I1(t),…,IM(t) representing switching loads. At observation time T, the voltage at a victim node v has drooped below Vdd:

ΔVv(T) = Vdd−Vv(T) > 0  (mV).

The blame assignment problem asks: how much of ΔVv(T) is attributable to each aggressor instance? That is, find scalars {cj}j=1M such that

ΔVv(T) = j=1Mcj,

where cj quantifies the contribution of current source j to the victim’s droop. Once computed, the top-K aggressors by |cj| identify the dominant switching noise contributors for design remediation.

## 1.2 Linearity Enables Exact Decomposition

The key enabler is linearity. Under the LTI RC model, the nodal voltage Vv(T) is a linear functional of the current excitations. By superposition, the response to the combined excitation I(t)=j​Ij(t) equals the sum of individual responses:

Vv(T) = Vv(0)(T)+j=1MVv(j)(T),

where Vv(j) is the voltage response to source j alone and Vv(0) is the homogeneous response from the initial condition and pad voltages.

Rather than running M separate forward simulations (prohibitively expensive for M>105 sources in production PDNs), the impulse response method computes all M contributions with a single backward-in-time evaluation of the network’s sensitivity kernel.

# 2. System Formulation

## 2.1 Nodal Equations

The complete set of nodal equations for the RC network is:

CdVdt+G V=− I(t),

where G is the conductance matrix (mS), C is the capacitance matrix (fF), V is the node voltage vector, and I(t) is the vector of injected load currents (mA). The negative sign follows the convention that positive current denotes a sink drawing current from the grid.

## 2.2 Reduced System

Pad nodes are held at constant voltage Vp=Vdd (Dirichlet boundary conditions). Eliminating the pad rows and columns via standard Schur complement reduction yields the reduced system on the Nu unknown (non-pad) nodes:

Cuu Vu+Guu Vu=−Iu(t),  Vu(0)=Vu(0),

where Iu(t) is the load current vector on the unknown nodes (positive = sink). The constant pad contribution through resistive coupling is absorbed into the equilibrium voltage and does not affect the time-varying blame decomposition. Both Guu and Cuu are symmetric positive semi-definite for a passive RC network; Guu is SPD when every unknown node has a resistive path to at least one pad. In the PDN, all tile-level capacitors are grounded, so Cuu is diagonal.

## 2.3 Unit System

The PDN uses internally consistent units: resistance in kΩ, conductance in mS, capacitance in fF, current in mA, voltage in V. This gives RC time constants in ns (kΩ× fF = ns). For time discretization, the time step is scaled to picoseconds: Δts=Δt×1012, ensuring dimensional consistency: G[mS]+C[fF]/Δts[ps]=mS.

# 3. Blame via Impulse Response

This section derives the blame decomposition using only linearity and the impulse response of the RC network. No variational calculus or Lagrange multipliers are needed.

## 3.1 Solution of the Reduced LTI System

The reduced ODE from Section 2.2 is a linear, time-invariant system. Its solution is given by the standard variation-of-parameters formula:

Vu(T)=Φ(T) Vu(0)−0TG(T−τ) Iu(τ) dτ,

where:

- Φ(s)=exp​−Cuu−1Guu s is the state transition matrix, describing how the homogeneous (unforced) system evolves.

- G(s)=Φ(s) Cuu−1 is the impulse response matrix (Green’s function) of the RC network.

The entry Gij(s) has a direct physical meaning: it is the voltage at node i measured s seconds after a unit impulse of current (1 mA⋅ps) is injected at node j. The impulse response satisfies:

Cuu dGds+Guu G(s)=0,  s>0,  G(0+)=Cuu−1.

Since Guu and Cuu are symmetric, so is G(s): the voltage at node i from an impulse at j equals the voltage at j from an impulse at i. This is the reciprocity property of passive RC networks.

## 3.2 Blame as Convolution

The current source vector decomposes as Iu(τ)=j=1Menj Ij(τ), where enj is the unit vector at the grid node nj where source j injects current. Substituting into the solution and projecting onto the victim node v:

Vv(T)=ev⊤Φ(T) Vu(0)⏟Vvhom(T) (IC contribution) − j=1M0TGv,nj(T−τ) Ij(τ) dτ.

The IR-drop contributed by source j is therefore:

cj=0TGv,nj(T−τ) Ij(τ) dτ.

This is a convolution of the impulse response with the source’s current waveform. The result is exact — it follows from nothing more than linearity and the definition of the impulse response.

Conservation. Summing over all sources:

j=1Mcj=Vvhom(T)−Vv(T)=ΔVv(T)−ΔVvhom(T).

For zero initial conditions (Vu(0)=Vdd⋅1), the homogeneous response maintains Vdd and the sum equals the total IR-drop ΔVv(T).

## 3.3 Efficient Evaluation: The Sensitivity Kernel

Computing the blame integral for all M sources naively would require evaluating M different entries Gv,nj(s) of the impulse response matrix. But there is a crucial simplification: all M blames share the same column of G.

Define the sensitivity kernel:

h(s)=G(s) ev,

the v-th column of the impulse response matrix. Then:

cj=0Thnj(T−τ) Ij(τ) dτ,

where hnj(s) is the nj-th component of h(s). All M blame integrals are computed from the single vector-valued function h(s).

The sensitivity kernel satisfies the homogeneous RC ODE with a jump initial condition:

Cuu dhds+Guu h(s)=0,  s>0,  Cuu h(0+)=ev.

Equivalently, extending h(s)=0 for s<0, the kernel satisfies the distributional ODE with a Dirac source:

Cuu dhds+Guu h(s)=ev δ(s).

The two forms are equivalent: integrating across s=0 gives Cuu[h(0+)−h(0−)]=ev, and since h(0−)=0 this recovers the jump condition Cuu h(0+)=ev. The Dirac form unifies the initial impulse with the dynamics into a single equation and maps naturally to the discrete formulation (Section 4.2).

Physically, h(s) is a standard RC decay from an impulse concentrated at the victim node. The backward sweep of the blame assignment algorithm is precisely the numerical evaluation of h(s) at discrete lag values s=0,Δt,2Δt,…

Impulse at victim v                    Sensitivity kernel h(s)
           |                               decays outward from v
    t=T    |  s=0: h(0) = C_uu^{-1} e_v   (concentrated at v)
           |
    t=T-dt |  s=dt: h(dt)                  (spread to neighbors)
           |
    t=T-2dt|  s=2dt: h(2dt)               (further spread)
           |
           v
    t=0    |  s=T: h(T)                    (decayed to near-zero)

    At each lag s = T - t_k:
      c_j += h_{n_j}(s) * I_j(t_k)

    Figure 1: The sensitivity kernel h(s) starts as a unit impulse at
    the victim and decays outward through the RC network. At each time
    step, the kernel value at each source's node is multiplied by the
    source's current to accumulate blame.

# 4. Discrete-Time Formulation (Backward Euler)

## 4.1 Forward Discretization

Discretize the reduced ODE (Section 2.2) using Backward Euler at uniform time step Δt (seconds). With Δts=Δt×1012 (ps), the discrete forward system is:

A Vun+1=−Iu(tn+1)+B Vun,

where:

A=Guu+CuuΔts,  B=CuuΔts.

The matrix A is SPD, admitting a sparse Cholesky factorization.

## 4.2 Discrete Impulse Response

Apply Backward Euler to the Dirac form of the sensitivity kernel ODE (Section 3.3), Cuu h+Guu h=ev δ(s). Denote the sampled continuous kernel at lag s=l⋅Δt by hl. With h−1=0:

CuuΔts(hl−hl−1)+Guu hl=evΔts δl,0,

where δl,0 is the Kronecker delta and 1/Δts is the Dirac-to-Kronecker conversion. Rearranging:

A hl=B hl−1+evΔts δl,0.

The continuous blame is an integral (∫h I dτ), but the discrete blame is a sum (∑λ I). Define the discrete kernel λl=Δts⋅hl to absorb the integration measure. Multiplying through by Δts:

A λl=B λl−1+ev δl,0.

At l=0: A λ0=ev, so λ0=A−1 ev. For l≥1: A λl=B λl−1.

Re-indexing to backward time notation (as used in the implementation), with λk≡λN−1−k where k now indexes forward time:

λN−1=A−1 ev,  A λk=B λk+1 (k=N−2,…,0).

This is the backward sweep: the same A factorization is reused at every step.

Verification from the discrete forward equations. The same result follows independently from differentiating the forward system A Vn+1=−Iun+1+B Vn. The sensitivity of the victim’s IR-drop to the forcing at step k is ev⊤(A−1B)N−1−kA−1. At the final step this is ev⊤A−1, so λN−1=A−1ev captures this sensitivity by symmetry of A, confirming the terminal condition from a purely discrete perspective.

## 4.3 Discrete Blame Formula

The discrete blame is:

cj=k=0N−1λk[nj] ⋅ Ij(tk).

No Δt factor. The Δts is already absorbed into λ: since λl=Δts⋅hl, the discrete sum k​λk⋅Ij(tk) approximates the continuous integral ∫hnj(T−τ) Ij(τ) dτ directly.

Side-by-side comparison:

| Continuous | Discrete (BE) |
| --- | --- |
| cj=0Thnj(T−τ) Ij(τ) dτ | cj=k=0N−1λk[nj]⋅Ij(tk) |
| Cuu h+Guu h=ev δ(s) | A λl=B λl−1+ev δl,0 |
| h(s) has units kΩ/ps | λk has units kΩ (= V/mA), absorbs Δts |

## 4.4 Unit Analysis

| Quantity | Symbol | Units | Derivation |
| --- | --- | --- | --- |
| System matrix | A | mS | Guu[mS]+Cuu[fF]/Δts[ps] |
| History matrix | B | mS | Cuu[fF]/Δts[ps] |
| Sensitivity kernel (discrete) | λk | kΩ (= V/mA) | From A−1 where [A]= mS |
| Source current | Ij(tk) | mA | PDN convention |
| Single-step contribution | λk[nj]⋅Ij | V | kΩ× mA |
| Blame (accumulated) | cj | V (reported as mV) | k​​ of above, ×1000 |

ALGORITHM: Backward Sweep (Discrete Impulse Response)
    ======================================================

    Input:  victim index v, time array [t_0, ..., t_{N-1}],
            factored A (LU or Cholesky), matrix B,
            current sources {I_j(t)}

    Output: blame {c_j} for each source j

    1. FACTOR A (or reuse from forward transient solve)
       lu_A = cholesky(A)             // O(nnz^{3/2}), one-time

    2. TERMINAL CONDITION (impulse response at lag 0)
       e_v = zeros(n_unknown)
       e_v[v] = 1.0
       lambda_next = lu_A.solve(e_v)  // O(nnz)

    3. INITIALIZE accumulators
       c[j] = 0   for all j

    4. BACKWARD SWEEP: k = N-1, N-2, ..., 0

       if k == N-1:
           lambda_current = lambda_next
       else:
           rhs = B * lambda_next       // sparse matvec, O(nnz)
           lambda_current = lu_A.solve(rhs)  // O(nnz)

       // Evaluate currents and accumulate (O(M))
       for each source j:
           c[j] += lambda_current[n_j] * I_j(t_k)

       swap(lambda_current, lambda_next)

    5. CONVERT to mV:  c[j] *= 1000

    Memory: O(n + M)  -- only 2 lambda vectors + M accumulators
    Time:   O(N * (nnz + M))  -- N back-solves + N * M accumulations

    Figure 2: Algorithmic flowchart. The LU factorization from the
    forward transient solve is reused (symmetric A). Only two lambda
    vectors are kept in memory at any time (O(n) swap trick).

# 5. Initial Conditions

## 5.1 Zero IC Mode

In zero IC mode, the initial condition is Vu(0)=Vdd⋅1 (zero IR-drop at t=0). The blame attributes the total IR-drop at the victim:

cjzero=k=0N−1λk[nj] ⋅ Ij(tk).

The sum j​cjzero=ΔVv(T) accounts for the entire droop from the pristine Vdd state.

When to use: When the engineer wants total accountability of all switching activity within the observation window. Appropriate when the memory window encompasses the full simulation from startup.

## 5.2 DC IC Mode

In DC IC mode, the initial condition is the DC operating point Vu(0)=VDC, computed by solving the static system:

Guu VDC=−IDC  (pad contribution implicit),

where IDC is the static (DC) component of the load currents. The blame then attributes only the incremental IR-drop above the DC baseline:

cjinc=k=0N−1λk[nj] ⋅ Ij(tk)−Ij,DC.

The DC baseline IR-drop is ΔVvDC=Vdd−VDC,v (mV).

## 5.3 Relationship Between Modes

Define the static contribution of source j:

cjstatic=k=0N−1λk[nj] ⋅ Ij,DC.

Then:

cjzero=cjinc+cjstatic.

This identity holds exactly by linearity. The total blame decomposes into an incremental part (switching above DC level) and a static part (constant background current). DC IC mode isolates switching noise from the static voltage map.

# 6. Per-Instance Blame Aggregation and Top-K Ranking

## 6.1 Aggregation Hierarchy

The blame formula (Section 4.3) computes blame at the level of individual current sources cj. In practice, multiple sources may be associated with a single design instance. The aggregation proceeds:

- Per-source: cj for each current source j.

- Per-node: cnode=j:node(j)=node​cj summing all sources at the same grid node.

- Per-instance: cinst=j∈inst​cj summing all sources belonging to the same physical instance (cell/macro).

The per-instance level is the most actionable for design: it tells the engineer which block to resize, move, or add decap near.

## 6.2 Self-Contribution

The victim’s own current sources (if any) are separated:

cself=j∈sources(v)​cj.

This quantifies how much of the droop is self-inflicted versus caused by remote aggressors.

## 6.3 Top-K Ranking

After aggregation, sort instances by |cinst| descending and report the top K. Each entry includes:

- Instance identifier

- Contribution in mV and as a percentage of total IR-drop

- List of constituent source names

- (Optional) Current waveform over the memory window

## 6.4 Attribution Efficiency

Define:

η=j=1McjΔVv(T).

For a linear system with the memory window encompassing all relevant history, η=1.0 exactly. Deviations from unity indicate truncation or numerical artifacts (see Section 8).

# 7. Algorithmic Complexity and Implementation Notes

## 7.1 Cost Breakdown

| Phase | Cost | Notes |
| --- | --- | --- |
| LU factorization of A | O(nnnz3/2) | One-time; shared with forward transient |
| Terminal condition solve | O(nnnz) | Single back-substitution |
| Each backward step | O(nnnz) | SpMV (B⋅λ) + back-sub |
| Current evaluation per step | O(M) | Vectorized PWL evaluation |
| Contribution accumulation per step | O(M) | Dot product per source |
| Total backward sweep | O(L⋅(nnnz+M)) | L = number of time steps |

For typical PDN sizes (n∼106 unknowns, M∼105 sources, L∼100 steps), the backward sweep is dominated by the L sparse triangular solves.

## 7.2 Memory

The backward sweep requires only two λ vectors (2×n doubles) via the swap trick, plus M scalar accumulators. Total: O(n+M). The LU factors (from the forward solve) are the dominant memory consumer.

## 7.3 LU Reuse

The system matrix A=Guu+Cuu/Δts is identical for the forward transient and backward sweep (since A⊤=A for symmetric Guu, Cuu). The implementation shares the cached factorization, avoiding a redundant O(nnnz3/2) factorization. This typically saves 200–500 MB of memory.

## 7.4 Vectorized Current Evaluation

Current sources are stored in a VectorizedCurrentSources structure that supports O(M) batch evaluation of all PWL/pulse waveforms at a given time t. The per-source evaluation returns individual source currents needed for the accumulation inner loop.

## 7.5 Spatial Filtering

When only nearby aggressors are of interest (e.g., within a spatial window around the victim), the candidate source set can be pruned a priori. This reduces the inner-loop cost from O(M) to O(Mwindow) per step. The sensitivity kernel λ is still computed over the full grid.

# 8. Practical Considerations and Sensitivity Analysis

## 8.1 Attribution Efficiency Deviations

The attribution efficiency η (Section 6.4) should be exactly 1.0 for a linear system when the memory window and source set are complete. In practice, η deviates due to:

- Memory window truncation: If the backward sweep starts at t0>0 rather than t=0, contributions from the interval [0,t0) are missed.

- Spatial window exclusion: Sources outside the spatial window are not counted, even though they contribute through the grid’s far-field coupling modes (see Appendix B).

- Numerical precision: Accumulation over many small terms introduces floating-point rounding errors, typically at the 10−10 level.

## 8.2 Sensitivity to Memory Window Length

The sensitivity kernel decays exponentially with lag. The spectral radius of the one-step propagator A−1B determines the decay rate:

ρ(A−1B)=maxk1/Δtsμk+1/Δts=τmaxτmax+Δts,

where τmax=1/μmin is the slowest RC time constant (see Appendix B for the spectral decomposition). After L steps, the residual kernel magnitude is bounded by:

∥λN−1−L∥≤∥λN−1∥⋅ρL.

To ensure truncation error below a fraction ϵ:

L>ln(1/ϵ)ln(1/ρ)≈τmaxΔts⋅ln(1/ϵ).

Rule of thumb: For ϵ=1% (η>0.99), set L⋅Δt>5⋅τmax. For ϵ=0.1%, use 7⋅τmax.

Attribution efficiency eta vs. memory window length
    (normalized to tau_max)

    eta
    1.00 |                              _______________
         |                         ____/
    0.95 |                    ____/
         |               ____/
    0.90 |          ____/
         |     ____/
    0.85 | ___/
         |/
    0.80 +----------+----------+----------+----------+-->
         0       2*tau      4*tau      6*tau      8*tau
                         Memory window length

    Figure 3: Attribution efficiency as a function of memory window
    length. The curve approaches 1.0 exponentially; 5*tau_max gives
    >99% efficiency for most PDN topologies.

## 8.3 Sensitivity to Time Step Δt

Stiff regime (τmax≪Δt): The kernel decays to near-zero in a single step (ρ≈0). Only the terminal step contributes:

cj≈λN−1[nj]⋅Ij(T)=(A−1ev)[nj]⋅Ij(T)≈(Guu−1ev)[nj]⋅Ij(T).

This converges to the static sensitivity Guu−1. In the stiff limit, a single matrix solve suffices.

Non-stiff regime (τmax≫Δt): Memory extends over many steps. Multiple past time steps contribute significantly. Reducing Δt improves temporal resolution at the cost of more backward steps.

Crossover: At Δt≈τmax, the ranking transitions from being dominated by recent/local activity to including historical/distant contributions.

## 8.4 Sensitivity to Spatial Window

Restricting the spatial window (Section 7.5) trades completeness for speed. The “leakage” — blame attributed to sources outside the window — is carried by the low-frequency eigenmodes of the RC network (see Appendix B for the spectral interpretation):

- Near-field modes (localized, fast decay): fully captured by a moderate window.

- Far-field modes (global, slow decay): extend across the grid. Sources outside the window contribute through these modes, and their blame is missed.

For PDNs with strong far-field coupling (sparse pad placement, uniform current loads), a large spatial window or no window is needed.

## 8.5 Sensitivity to Instance Grouping

The top-K ranking can change depending on the aggregation level:

- Per-node ranking: Fine-grained. Two sources on different nodes of the same instance appear as separate entries.

- Per-instance ranking: Coarse-grained. An instance with many small-contribution sources may rank higher than a single large-contribution source.

Recommendation: Report both per-instance blame and the number of constituent sources. An instance with 50 sources each contributing 0.1 mV (total 5 mV) has a qualitatively different character than one source contributing 5 mV.

# 9. Validation

The blame assignment can be validated through four independent checks:

## 9.1 Aggressor-Only Simulation

For each top aggressor j, run a forward transient with only source j active (all others zeroed), starting from Vu(0)=Vdd. The IR-drop at the victim at time T should match the blame:

ΔVv(j)(T)≈cj.

The match is exact when the memory window encompasses the full simulation interval.

## 9.2 Superposition Check

Run the full forward transient and verify:

j=1Mcj≈ΔVv(T),  η≈1.0.

## 9.3 Stiff-System Convergence

For τmax≪Δt, verify that the dynamic blame matches the static sensitivity Guu−1ev⋅Ij(T).

## 9.4 DC Mode Decomposition

Verify the identity from Section 5.3:

cjinc+cjstatic=cjzero  ∀ j.

This holds to machine precision by linearity.

# 10. Conclusion

## Summary

This report derived the impulse-response-based method for exact per-instance decomposition of dynamic IR-drop in LTI RC power distribution networks. The key results are:

- Exact decomposition: The victim’s IR-drop is a sum of convolutions of the impulse response with each source’s current waveform. Linearity guarantees exactness.

- Single backward sweep: The sensitivity kernel h(s) — a single column of the impulse response matrix — captures all M aggressor contributions. Computational cost: O(L⋅(nnnz+M)) with O(n+M) memory.

- Physical interpretation: h(s) is the RC network’s response to an impulse at the victim, decaying outward with distance and time. Nearby, recent switching dominates; distant, historical switching contributes through slow global modes (Appendix B).

- Generalization: The Lagrangian adjoint formulation (Appendix C) recovers the same result for LTI systems and extends to nonlinear and time-varying networks where the impulse response approach does not apply.

## Limitations

- Memory truncation: Contributions from before the backward sweep window are invisible.

- No inductance: Package-level Ldi/dt droop is not captured. Extending to RLC requires a non-symmetric formulation.

- Flat formulation: The distributed DDM extension (tile-based Schur complement) is implemented but documented separately.

## Extensions

- Trapezoidal rule: Uses A=Guu+2Cuu/Δts, B=2Cuu/Δts−Guu.

- Distributed DDM: Per-tile local sweeps with a coordinator interface solve.

- RLC networks: Coupled backward sweep on voltage and current variables.

# Appendix A: Notation Table

| Symbol | Description | Units |
| --- | --- | --- |
| Guu | Reduced conductance matrix (unknowns only) | mS |
| Cuu | Reduced capacitance matrix (unknowns only) | fF |
| A | System matrix: Guu+Cuu/Δts | mS |
| B | History matrix: Cuu/Δts | mS |
| Vu | Voltage vector at unknown nodes | V |
| Vdd | Supply voltage | V |
| Ij(t) | Current drawn by source j at time t | mA (positive = sink) |
| ev | Unit vector at victim index v | dimensionless |
| Φ(s) | State transition matrix: exp(−Cuu−1Guu s) | dimensionless |
| G(s) | Impulse response matrix: Φ(s) Cuu−1 | kΩ/ps |
| h(s) | Sensitivity kernel: G(s) ev (v-th column) | kΩ/ps |
| λk | Discrete sensitivity kernel at step k (=Δts⋅hk, absorbs integration measure) | kΩ (= V/mA) |
| cj | Blame contribution of source j | mV |
| ΔVv(T) | IR-drop at victim: Vdd−Vv(T) | mV |
| η | Attribution efficiency: ∑cj/ΔVv(T) | dimensionless |
| Δt | Time step | seconds |
| Δts | Scaled time step: Δt×1012 | ps |
| T | Observation time | seconds |
| L | Number of backward steps (memory window) | integer |
| τk=1/μk | k-th RC time constant | ns |
| ρ | Spectral radius of A−1B | dimensionless |
| ϕk | k-th eigenmode of Cuu−1Guu | (normalized) |
| nj | Grid node at which source j injects current | (node ID) |
| Nu | Number of unknown nodes | integer |
| M | Number of current sources | integer |

# Appendix B: Spectral Interpretation

## B.1 Eigenmode Decomposition

Consider the generalized eigenproblem:

Guu ϕk=μk Cuu ϕk,  ϕk⊤Cuu ϕl=δkl.

The eigenvalues μk>0 set the RC time constants τk=1/μk. The eigenmodes ϕk are spatial patterns on the grid. The impulse response decomposes as:

G(s)=k=1Nuϕk ϕk⊤ e−μks.

Substituting into the blame integral (Section 3.2):

cj=k=1Nuϕk(v) ϕk(nj) 0Te−μk(T−τ) Ij(τ) dτ⏟filtered current through mode k.

## B.2 Spatial-Temporal Factorization of Blame

The spectral form reveals a fundamental factorization:

cj=k​ϕk(v) ϕk(nj)⏟spatial coupling × Ij,k(T)⏟temporally filtered current.

Low-frequency modes (small μk, large τk): - Spatially smooth, extending across the grid - Decay slowly (e−μks persists for large s) - Carry far-field blame: distant aggressors contribute through these global modes - Integrate long current history

High-frequency modes (large μk, small τk): - Spatially localized, concentrated near the source - Decay rapidly (e−μks→0 quickly) - Carry near-field blame: only local, recent switching events contribute - Respond primarily to the current at or near time T

This explains why distant aggressors contribute to IR-drop primarily through slow, global RC modes, and nearby aggressors dominate due to fast, local modes with large ϕk(v)⋅ϕk(nj) products.

Mode k=1 (global, slow decay):
     ~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~  tau_1 = large
     Spatially smooth across entire grid

    Mode k=2 (intermediate):
     ~~~~  ~~~~  ~~~~  ~~~~  ~~~~          tau_2 = moderate
     Half-wavelength oscillation

    Mode k=N (local, fast decay):
     ___|___                               tau_N = small
     Sharp peak near source location

    Decay envelopes:
    exp(-mu_k * s)
      |
    1 |---___  k=1 (slow)
      |      ---___
      |   ----___   k=2
      |          ---___
      |  -------___     k=N (fast)
      |___________________|______> s (time lag)
      0          5*tau_1

    Figure B.1: Spatial eigenmodes (top) and their temporal decay
    envelopes (bottom). Low-frequency modes are spatially global and
    persist in time; high-frequency modes are spatially local and
    decay rapidly.

# Appendix C: Lagrangian Adjoint — Generalization

## C.1 Motivation: Beyond LTI

The impulse response approach of Section 3 exploits two properties of the PDN model: linearity (superposition) and time-invariance (the impulse response G(s) depends only on the lag s=T−τ, not on τ itself). For systems that violate either property — nonlinear circuit elements, time-varying topology, or constrained optimization over design parameters — the impulse response does not exist as a fixed kernel.

The Lagrangian adjoint method generalizes blame assignment to these settings. For our LTI RC PDN, it recovers exactly the same result as Section 3, providing an independent validation.

## C.2 Continuous Lagrangian Derivation

Working on the reduced system (Section 2.2), define the output of interest:

J=ev⊤Vu(T).

Introduce the adjoint variable λ(t) as a Lagrange multiplier enforcing the forward ODE. The Lagrangian is:

L=ev⊤Vu(T)+0Tλ(t)⊤−Iu(t)−Cuu Vu(t)−Guu Vu(t) dt.

Integrate the λ⊤Cuu Vu term by parts:

0Tλ⊤Cuu Vu dt=[λ⊤Cuu Vu]0T−0Tλ⊤Cuu Vu dt.

Substituting and collecting terms involving Vu:

L=(ev−Cuuλ(T))⊤Vu(T)+λ(0)⊤Cuu Vu(0)+0Tλ⊤Cuu−λ⊤GuuVu dt−0Tλ⊤Iu dt.

Setting the variation δL/δVu=0 yields:

Adjoint ODE:

−Cuu λ(t)+Guu λ(t)=0,  t∈[0,T).

Terminal condition (from the boundary term at T):

Cuu λ(T)=ev.

The blame of source j is obtained from the variation with respect to Ij:

cj=0Tλnj(τ) Ij(τ) dτ.

## C.3 Equivalence with the Impulse Response

Claim: λ(τ)=h(T−τ) where h(s) is the sensitivity kernel from Section 3.3.

Proof. The sensitivity kernel satisfies:

Cuu dhds+Guu h(s)=0,  Cuu h(0+)=ev.

Define λ(τ)=h(T−τ). Then λ(τ)=−h′(T−τ), so:

−Cuu λ(τ)+Guu λ(τ)=Cuu h′(T−τ)+Guu h(T−τ)=0.

And at τ=T: Cuu λ(T)=Cuu h(0+)=ev.

So λ and h satisfy the same ODE with the same boundary condition, just parameterized differently: λ runs backward in physical time τ, while h runs forward in lag s=T−τ. The blame integrals are identical. ▫

## C.4 Discrete Lagrangian Derivation

The discrete Lagrangian over time steps n=0,…,N−1:

Ld=ev⊤VuN−1+n=0N−2λn⊤−Iun+1+B Vun−A Vun+1.

Taking variations with respect to Vuk for 0<k<N−1 gives the discrete adjoint recurrence:

A⊤λk−1=B⊤λk.

For symmetric A and B (passive RC): A λk=B λk+1, with terminal condition A λN−1=ev. This is identical to the discrete impulse response recurrence (Section 4.2), confirming consistency.

The discrete Lagrangian also explains the no-Δt-factor: varying Ld with respect to Ij(tk) gives ∂Ld/∂Ij(tk)=λk[nj], so the blame cj=k​λk[nj]⋅Ij(tk) has no explicit Δt.

## C.5 When the Lagrangian is Needed

| Setting | Impulse Response | Lagrangian Adjoint |
| --- | --- | --- |
| LTI RC (this report) | Primary method | Equivalent, provides validation |
| Nonlinear elements | Not applicable (no superposition) | Required |
| Time-varying systems | Kernel depends on both t and τ | Handles naturally |
| Optimization (decap placement) | Not applicable | Natural framework |
| RLC with inductance | Requires non-symmetric extension | Handles naturally |

For our LTI RC PDN, the impulse response derivation (Section 3) is complete and sufficient. The Lagrangian is not needed but validates the result from a different mathematical angle and provides the generalization path for future extensions.